#include <Servo.h>

// Digital Output pins for LEDs

//-----------------------------

const int blueLED = 7;

const int uvLED1 = 13;

const int uvLED2 = 12;

const int grnLED1 = 11;

const int grnLED2 = 10;

const int orgLED1 = 9;

const int orgLED2 = 8;

// Analog/Digital Input pins

//-----------------------------------------

const int LDR = A0;

const int mic = A1;

const int button = 5;

// Analog Sensor Values

//---------------------

int LDRval;

int micVal;

const int threshold = 800;

// Servo control stuff

//--------------------

Servo myservo;  // create servo object to control a servo

// twelve servo objects can be created on most boards

int pos = 0;    // variable to store the servo position

// Generally, you should use "unsigned long" for variables that hold time

// The value will quickly become too large for an int to store

unsigned long previousMillis = 0;        // will store last time LED was updated

// constants won't change:

int timer = 0;

unsigned long last_event = 0;

boolean blueLED_state = false;

//----------------------------------------------------------------------------------------

void setup() {

Serial.begin(9600);

myservo.attach(3);

for (int ledPin = 7; ledPin < 14; ledPin++) {

pinMode(ledPin, OUTPUT);

digitalWrite(ledPin, LOW);

}

pinMode(LDR, INPUT);

pinMode(mic, INPUT);

}

//-----------------------------------------------------------------------------------------

void loop() {

// check to see if it's time to blink the LED; that is, if the difference

// between the current time and last time you blinked the LED is bigger than

// the interval at which you want to blink the LED.

unsigned long currentMillis = millis();

timer = analogRead(LDR);

timer = map(timer, 0, 1023, 400, 0);

Serial.print(timer);

for (int ledPin = 7; ledPin < 14; ledPin++) {

// turn the pin on:

digitalWrite(ledPin, HIGH);

delay(timer);

// turn the pin off:

digitalWrite(ledPin, LOW);

micVal = analogRead(mic);

if (micVal > threshold)

{

digitalWrite(blueLED, HIGH);

delay(1000);

digitalWrite(blueLED, LOW);

}

else {

digitalWrite(blueLED, LOW);

}

}

int pb = digitalRead(button);

if (pb == HIGH) {

for (pos = 0; pos <= 180; pos += 1) { // goes from 0 degrees to 180 degrees

// in steps of 1 degree

myservo.write(pos);              // tell servo to go to position in variable 'pos'

delay(15);                       // waits 15 ms for the servo to reach the position

}

for (pos = 180; pos >= 0; pos -= 1) { // goes from 180 degrees to 0 degrees

myservo.write(pos);              // tell servo to go to position in variable 'pos'

delay(15);                       // waits 15 ms for the servo to reach the position

}

}

}